NOTE: Use of internet is permitted only to access software website, calculators are permitted and your answers must include worked solutions. If you require extra sheet(s) please write your name and student number at the top of each additional sheet.

https://logic.ly/

Part A

Objective

Understand various logic gates

- Using Logic.ly create a circuit file and label Gate Symbols, Functional Notation and truth table for the gates listed in the table below:

Inputs; Output(s) Truth Table. A; B; AND; OR; XOR; NAND; NOR. 0; 0; 0; 0; 0; 1; 1. 0; 1; 0; 1; 1; 1; 0. 1; 0; 0; 1; 1; 1; 0. 1; 1; 0; 1; 0; 0; 0. Gate Symbol. Functional Notation; A ^ B; A B; (A^B); ¬A  ¬B.

- Construct the logic circuit below using Lab Logic and detail the functional notation and truth table for a two input AND gate. Switches; AND Gate; LED Output.

Part B

Objective

Understand the construction of Logic Gates from various combinations of logic gates

- Using Lab Software prove that combining and AND and a NOT constructs a NAND Gate NOTE: solution must be demonstrated to Lecturer prior to leaving laboratory

Demonstrated to lecturer; Yes; No..

- Prove using Lab Software and a truth table that the circuit below is a XOR Gate. Place an LED on the output of the constructed circuit. NOTE: solution must be demonstrated to Lecturer prior to leaving laboratory Inputs. A; B; XOR. 0; 0; 0. 0; 1; 1. 1; 0; 1. 1; 1; 0. Demonstrated to lecturer; Yes; No..

- Prove DeMorgan’s Law ¬(A  B) = ¬A  ¬B using Lab Software

Demonstrated to lecturer; Yes; No..

Hand up this practical report at the end of session and ensure it has been checked

| Student Name | Cillían Vickers | Student Number | C00263779 |
| --- | --- | --- | --- |
| Date | 24/11/21 | Checked |  |
| Group | A / B |  |  |